SVKM’s NMIMS Mukesh Patel School of Technology Management & Engineering Shirpur AIoT-Based Crop Recommendation System. (Kisan Mitra) Submitted By: Akshat Katiyar, E-272 Meghashayam Pawar, E-301 Saksham Hedaoo, E-297 Course: BTECH CS Academic Year:-2024-25 Faculty mentor: Dr. Bhushan Inje (Assistant Professor) MPSTME, NMIMS Shirpur Campus ARTIFICIAL INTELLIGENCE OF THINGS PROJECT REPORT

—

# Abstract

Kisan Mitra is a full-stack Artificial Intelligence of Things (AIoT) system designed to assist Indian farmers in making data-driven crop selection decisions. The system integrates a Wokwi-simulated ESP32 microcontroller equipped with a DHT22 temperature-humidity sensor and five potentiometers representing soil nutrient and environmental parameters. Real-time sensor data is transmitted securely via MQTT over TLS to a HiveMQ Cloud broker, consumed by a Flask-based backend, processed through a trained machine learning classifier, and further refined by Google's Gemini 2.5 Flash generative AI model. The system outputs the top three ranked crop recommendations with cost estimates, harvest timelines, and rotation-aware advisory. Results are displayed on both a custom HTML/JavaScript web dashboard and a Blynk IoT mobile dashboard, providing farmers with an accessible, dual-interface precision agriculture tool.

# 1. Introduction

Agriculture is the backbone of the Indian economy, employing over 50% of the workforce. Despite this, smallholder farmers face persistent challenges: lack of access to expert agronomic advice, inability to afford soil testing, and poor crop selection decisions that lead to yield loss and financial hardship.

Kisan Mitra ("Farmer's Friend" in Hindi) addresses these challenges by combining three modern technology paradigms into a single, accessible platform:

- Internet of Things (IoT) — real-time soil and weather data collection via simulated ESP32 sensors

- Machine Learning (ML) — a trained Random Forest classifier predicts the most suitable crops

- Generative AI — Google Gemini 2.5 Flash provides context-aware, cost-conscious recommendations

The system is designed for practical demonstration using Wokwi simulation, HiveMQ Cloud MQTT broker, and Blynk IoT dashboard — making it deployable without physical hardware while maintaining production-level architecture.

# 2. Objectives

The primary objectives of this project are:

- To simulate a real IoT sensor network using Wokwi ESP32 with DHT22 and potentiometer-based soil sensors

- To establish secure MQTT communication between IoT devices and a cloud backend via HiveMQ

- To apply a trained ML model for crop probability prediction based on soil and weather parameters

- To use Generative AI (Gemini) for producing farmer-friendly, seasonally-aware crop recommendations

- To implement crop rotation awareness using historical crop data stored in SQLite

- To provide dual dashboards — a web dashboard and a Blynk IoT dashboard — for maximum accessibility

# 3. System Architecture

The Kisan Mitra system follows a layered AIoT architecture consisting of four distinct layers:

## 3.1 IoT Sensing Layer

The sensing layer is implemented using a Wokwi-simulated ESP32 DevKit V1 microcontroller. The simulation includes the following components:

| Component | Parameter | Range / Description |
| --- | --- | --- |
| DHT22 Sensor | Temperature | 8°C – 44°C (real sensor simulation) |
| DHT22 Sensor | Humidity | 14% – 100% relative humidity |
| Potentiometer 1 | Nitrogen (N) | 0 – 140 mg/kg soil nutrient |
| Potentiometer 2 | Phosphorus (P) | 5 – 145 mg/kg soil nutrient |
| Potentiometer 3 | Potassium (K) | 5 – 205 mg/kg soil nutrient |
| Potentiometer 4 | Soil pH | 3.5 – 10.0 pH scale |
| Potentiometer 5 | Rainfall | 20 – 300 mm annual average |

The ESP32 reads all sensors every 800ms and publishes JSON payloads to the HiveMQ Cloud MQTT broker over TLS (port 8883), matching the publish frequency of a production IoT sensor node.

## 3.2 Communication Layer

MQTT (Message Queuing Telemetry Transport) protocol is used for lightweight, reliable IoT communication. The system uses HiveMQ Cloud as the managed broker with the following configuration:

| Parameter | Value |
| --- | --- |
| Broker | HiveMQ Cloud (EU Region) |
| Protocol | MQTT over TLS — port 8883 (MQTT) / 8884 (WebSocket) |
| Publish Topic | farm/sensor/data |
| Command Topic | farm/sensor/command |
| QoS Level | QoS 1 — at least once delivery |
| Payload Format | JSON — {N, P, K, temperature, humidity, ph, rainfall, source} |

## 3.3 Backend Processing Layer

The Flask-based Python backend handles all business logic including authentication, sensor data aggregation, ML inference, and AI recommendation generation. Key modules include:

| Module | Responsibility |
| --- | --- |
| mqtt_service.py | Subscribes to HiveMQ, maintains a rolling 50-message history, averages 3.5s of readings, falls back to simulation if MQTT unavailable |
| ml_model.py | Loads pre-trained Random Forest classifier and StandardScaler, returns top-N crops with probability scores from averaged sensor data |
| gemini_service.py | Calls Gemini 2.5 Flash API with crop candidates, region, season, crop history, and area — returns 3 ranked recommendations with cost breakdown |
| database.py | SQLite ORM layer — manages users, authentication (bcrypt), and crop history with schema migration support |
| blynk_service.py | Pushes live sensor readings and AI recommendations to Blynk Cloud via HTTP API in non-blocking daemon threads |
| auth.py | Flask Blueprint for user registration, login, logout with session-based authentication and password hashing |
| routes.py | REST API endpoints for sensor data, recommendations, crop history CRUD operations with authentication middleware |

## 3.4 Presentation Layer

The system provides two separate dashboard interfaces for maximum accessibility:

### 3.4.1 Web Dashboard (HTML/CSS/JavaScript)

- Login and registration pages with session-based authentication

- Live sensor readings panel showing all 7 parameters with source indicator (wokwi/simulated)

- Recommendation form accepting region name and field area in hectares

- AI recommendation cards showing crop name, reason, harvest timeline, and cost breakdown

- Crop history table with delete functionality for managing rotation records

### 3.4.2 Blynk IoT Dashboard

- 7 Gauge widgets (V0–V6) displaying live sensor values from IoT layer

- Label widgets (V7–V11) showing top 3 AI-recommended crops and details

- Last Updated timestamp widget (V12) showing real-time data freshness

- Accessible via Blynk mobile app (iOS/Android) and web browser

# 4. Data Flow

The end-to-end data flow through the Kisan Mitra system follows these steps:

| Step | Component | Action |
| --- | --- | --- |
| 1 | Wokwi ESP32 | Reads DHT22 (temp/humidity) and 5 potentiometers (N,P,K,pH,Rain) every 800ms |
| 2 | ESP32 → HiveMQ | Publishes JSON payload to farm/sensor/data over MQTT TLS (port 8883) |
| 3 | HiveMQ → Flask | Backend subscriber receives message, stores in rolling deque, pushes to Blynk gauges (V0–V6) |
| 4 | Farmer action | Farmer enters region and area on web dashboard, clicks Get Recommendation |
| 5 | Sensor averaging | Backend averages 3.5 seconds of MQTT readings to reduce noise |
| 6 | ML inference | StandardScaler transforms features; Random Forest returns top 5 crops with confidence scores |
| 7 | Gemini AI | Top 5 crops + region + season + crop history + area sent to Gemini 2.5 Flash; returns top 3 with costs |
| 8 | Dual output | Recommendations shown on web dashboard AND pushed to Blynk labels (V7–V11) |

# 5. Machine Learning Model

## 5.1 Dataset

The ML model is trained on the Crop_recommendation.csv dataset containing 2,200 records across 22 crop types. Each record contains seven features: Nitrogen (N), Phosphorus (P), Potassium (K), Temperature, Humidity, pH, and Rainfall, along with the recommended crop label.

## 5.2 Model Architecture

A Random Forest classifier was selected due to its robustness to noisy sensor data, ability to handle mixed feature scales, and excellent multi-class classification performance. The model outputs probability distributions across all 22 crop classes, enabling ranked crop selection.

| Property | Value |
| --- | --- |
| Algorithm | Random Forest Classifier (scikit-learn) |
| Preprocessing | StandardScaler — zero mean, unit variance normalization |
| Output | Probability scores for 22 crop classes; top-N selection |
| Model file | crop_model.joblib (3.5 MB) |
| Scaler file | scaler.joblib (1 KB) |
| Inference time | < 5ms per prediction (loaded once at startup) |

## 5.3 Crops Covered

The model covers 22 crop types including: Rice, Maize, Chickpea, Kidney Beans, Pigeon Peas, Moth Beans, Mung Bean, Black Gram, Lentil, Pomegranate, Banana, Mango, Grapes, Watermelon, Muskmelon, Apple, Orange, Papaya, Coconut, Cotton, Jute, and Coffee.

# 6. AI Recommendation Engine

## 6.1 Gemini Integration

Google Gemini 2.5 Flash is used as the generative AI backbone. The system constructs a structured prompt containing:

- Top 5 ML-predicted crops with confidence percentages

- Farmer's region and field area in hectares

- Current month, date, and season (Kharif/Rabi/Pre-Monsoon/Post-Monsoon)

- Last 5 crops grown (from SQLite history) to enforce crop rotation

Gemini returns a strictly formatted JSON array of exactly 3 crop recommendations, each containing: crop name, farmer-friendly reason (2-3 sentences), harvest duration in days, planting month, harvest month, planting cost per hectare in INR, and total cost for the specified area.

## 6.2 Reliability Features

- Three Gemini API keys configured in round-robin rotation to handle rate limits

- Automatic key switching on HTTP 429 (quota exceeded) errors

- Graceful fallback to top ML prediction if all API keys are exhausted

- JSON validation with automatic field population for missing keys

## 6.3 Crop Rotation Awareness

The system maintains a per-user crop history in SQLite. The last 5 grown crops are included in the Gemini prompt with explicit instruction to avoid recommending recently grown crops. This enforces scientifically sound crop rotation practices automatically.

# 7. Database Design

A lightweight SQLite database is used for user management and crop history tracking, requiring no separate database server installation.

## 7.1 Users Table

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-incrementing primary key |
| name | TEXT | Farmer's full name |
| email | TEXT UNIQUE | Login credential (unique) |
| password_hash | TEXT | Bcrypt hashed password |
| region | TEXT | Farmer's geographic region |
| created_at | DATETIME | Registration timestamp |

## 7.2 Crop History Table

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-incrementing primary key |
| user_id | INTEGER FK | Foreign key to users table |
| crop_name | TEXT | Name of recommended/saved crop |
| reason | TEXT | AI-generated recommendation reason |
| harvest_duration | TEXT | e.g. 90-120 days |
| planting_month | TEXT | Ideal planting month |
| planting_cost_per_hectare | TEXT | Cost in INR per hectare |
| total_cost | TEXT | Total cost for specified area |
| date_recorded | DATETIME | Timestamp of recommendation |

# 8. REST API Reference

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /api/auth/register | Register a new farmer account |
| POST | /api/auth/login | Login and create session |
| POST | /api/auth/logout | Destroy current session |
| GET | /api/sensor/latest | Get most recent sensor reading |
| POST | /api/sensor/refresh | Send refresh command to ESP32 via MQTT |
| POST | /api/recommend | Get AI crop recommendations (region + area required) |
| POST | /api/recommend/save | Save a selected recommendation to crop history |
| GET | /api/history | Retrieve all crop history entries for user |
| DELETE | /api/history/{id} | Delete a specific crop history entry |

# 9. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| IoT Hardware | Wokwi ESP32 + DHT22 | Sensor simulation (temperature, humidity) |
| IoT Hardware | Potentiometers (x5) | Simulate N, P, K, pH, Rainfall sensors |
| Firmware | Arduino C++ (ESP32) | Sensor reading, JSON serialization, MQTT publish |
| Messaging | MQTT / HiveMQ Cloud | Secure IoT message broker (TLS) |
| Backend | Python / Flask | REST API, business logic, session management |
| ML | scikit-learn | Random Forest classifier for crop prediction |
| AI | Google Gemini 2.5 Flash | Generative AI for recommendation advisory |
| Database | SQLite | User data, authentication, crop history |
| Web Frontend | HTML / CSS / JavaScript | Browser-based farmer dashboard |
| IoT Dashboard | Blynk Cloud | Mobile/web IoT dashboard with gauges |
| Libraries | paho-mqtt, joblib, requests | MQTT client, model loading, HTTP calls |

# 10. Key Features & Innovations

## 10.1 Intelligent Fallback System

The system is designed to remain functional even when the IoT layer is offline. If the MQTT broker is unreachable or the Wokwi simulator is not running, the backend automatically falls back to a physics-based random sensor simulation using realistic value ranges derived from the training dataset. This ensures uninterrupted service during demonstrations and development.

## 10.2 Sensor Data Averaging

Rather than using a single instantaneous sensor reading, the system collects and averages 3.5 seconds of MQTT messages before passing data to the ML model. This reduces the impact of sensor noise and transient fluctuations, producing more stable and accurate crop recommendations.

## 10.3 Dual Dashboard Architecture

The system simultaneously maintains two dashboards: a full-featured web dashboard for detailed interaction (registration, history management, recommendation details) and a Blynk IoT dashboard for real-time monitoring. Both dashboards update simultaneously when new sensor data arrives or recommendations are generated.

## 10.4 Crop Rotation Enforcement

The AI recommendation engine actively considers the farmer's crop history when generating recommendations. The last five crops grown on the farm are included in the Gemini prompt with explicit instructions to avoid recommending recently grown crops, promoting soil health through scientifically proven crop rotation practices.

## 10.5 Cost-Aware Recommendations

Unlike standard crop recommendation systems that only consider agronomic suitability, Kisan Mitra instructs Gemini to provide realistic planting cost estimates per hectare and total cost for the farmer's specific field area. This enables direct financial planning alongside crop selection.

# 11. Results & Demonstration

## 11.1 System Performance

| Metric | Result |
| --- | --- |
| MQTT message frequency | 800ms publish interval from Wokwi |
| Sensor averaging window | 3.5 seconds (approx. 4-5 readings) |
| ML inference time | < 5ms per prediction |
| Gemini API response time | 2-4 seconds typical |
| End-to-end recommendation time | ~5-7 seconds total |
| Blynk dashboard update latency | < 1 second after MQTT receipt |
| Crop classes supported | 22 crops across Kharif, Rabi, and Zaid seasons |

## 11.2 Demo Flow

- Start Wokwi simulation — ESP32 begins publishing sensor JSON to HiveMQ every 800ms

- Verify on HiveMQ Web Client — messages appear on farm/sensor/data topic

- Start Flask backend — MQTT subscriber connects, ML model loads, server starts on port 5000

- Open Blynk dashboard — Gauge widgets update in real-time with sensor readings from Wokwi

- Turn Wokwi potentiometers — sensor values change live on both Blynk and web dashboard

- Submit recommendation request — enter region and area, click Get Recommendation

- View results — top 3 crops appear on web dashboard AND Blynk labels V7-V11 simultaneously

# 12. Conclusion

Kisan Mitra successfully demonstrates the practical application of AIoT technologies in precision agriculture. By combining IoT sensor simulation (Wokwi ESP32), cloud-based message brokering (HiveMQ MQTT), machine learning inference (Random Forest), and generative AI advisory (Gemini 2.5 Flash), the system provides Indian farmers with actionable, cost-aware crop recommendations in under 10 seconds.

The dual-dashboard architecture — serving both a web browser and the Blynk IoT mobile platform — ensures accessibility across different user contexts. The intelligent fallback mechanism and sensor averaging provide production-grade reliability, while the crop rotation enforcement promotes sustainable farming practices.

The modular architecture ensures that each component — IoT, ML, AI, and UI — can be independently upgraded or replaced as technology evolves, making Kisan Mitra a scalable foundation for future precision agriculture applications.

Future Enhancements Physical ESP32 deployment with real soil sensors (EC, NPK probes) Multilingual support in regional Indian languages (Hindi, Marathi, Telugu) Weather API integration for dynamic rainfall data Market price integration for yield value estimation Offline ML inference on-device using TensorFlow Lite

# References

[1] Atkar, S. et al. (2020). Crop Recommendation Dataset. Kaggle. https://www.kaggle.com/datasets/atharvaingle/crop-recommendation-dataset

[2] Google DeepMind. (2024). Gemini 2.5 Flash Technical Report. Google AI.

[3] HiveMQ. (2024). HiveMQ Cloud Documentation — MQTT Broker Service. https://docs.hivemq.com/hivemq-cloud/

[4] Wokwi. (2024). Wokwi Simulator Documentation — ESP32 Simulation. https://docs.wokwi.com/

[5] Blynk Inc. (2024). Blynk IoT Platform Documentation. https://docs.blynk.io/

[6] Pedregosa, F. et al. (2011). Scikit-learn: Machine Learning in Python. JMLR, 12, 2825-2830.

[7] O'Leary, N. (2018). PubSubClient — MQTT Library for Arduino. https://github.com/knolleary/pubsubclient

[8] Pallets Team. (2023). Flask Web Framework Documentation. https://flask.palletsprojects.com/

# 13. Literature review

| Se ria l No | Author Name | Year | Title | Challenge | Solution | Advantages | Disadvantages | Link |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MY Shams et al. | 2024 | Enhancing crop recommendat ion systems with explainable AI | Lack of transparenc y in AI recommend ations | XAI- CROP ensemble ML providing interpret able insights | Builds farmer trust and improves adoption | Higher computational overhead | https://lin k.springer. com/articl e/10.1007 /s00521- 023- 09391-2 |
| 2 | Motamedi et al. | 2024 | A predictive analytics model with Bayesian- Optimized ensemble crop recommendat ion | Need for more reliable and accurate crop recommend ation | Bayesian optimize d ensemble ML classifica tion framewo rk | High predictive performanc e with multi-class classificatio n | Requires careful tuning and quality datasets | https://w ww.science direct.com /science/a rticle/pii/S 27726622 24001206 |
| 3 | MSB Alam et al. | 2025 | An approach for crop recommendat ion with uncertainty quantification | Uncertainty and risk in automated crop recommend ations | Entropy- based uncertain ty quantific ation + ensemble ML | Provides confidence- aware recommend ations | Sensitive to noise in agricultural datasets | https://w ww.science direct.com /science/a rticle/pii/S 25901230 25015750 |
| 4 | H Kumari et al. | 2023 | A crop recommendat ion system for better sustainability | Choosing sustainable crops under varying soil conditions | Machine learning techniqu es forecasti ng ideal crops | Supports sustainabilit y and better resource use | Mainly conceptual; limited field deployment | https://dig ital- library.thei et.org/doi/ abs/10.10 49/icp.202 3.1512 |
| 5 | Y Mahale et al. | 2024 | Crop recommendat ion and forecasting system for smart agriculture | Need integrated crop forecasting with recommend ation | AI-based recomme ndation using soil and weather patterns | Improves crop planning decisions | Requires continuous environmental data collection | https://lin k.springer. com/articl e/10.1007 /s43621- 024- 00292-5 |
| 6 | S Shastri et al. | 2025 | Advancing crop recommendat ion with supervised ML and XAI | Low interpretabil ity of ML- based crop advisory systems | Gradient Boost + SHAP explanati ons for crop selection | High accuracy with explainabilit y | Needs large- scale training datasets | https://lin k.springer. com/articl e/10.1007 /s00521- 024- 09512-3 |
| 7 | M Baishya et al. | 2025 | TinyML based crop recommendat | Crop recommend ation on | TinyML deploym ent with | Works offline with low power | Limited crop diversity in training data | https://w ww.science direct.com |

|  |  |  | ion system for precision agriculture | low- resource edge devices | optimize d feature selection | consumptio n |  | /science/a rticle/pii/S 27723755 25004782 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | A Bhola et al. | 2025 | Farm-level smart crop recommendat ion framework using ML | Need localized crop recommend ation at farm level | ANN- based filtration and decision support system | Supports farmer- centric decision making | Region-specific tuning required | https://lin k.springer. com/articl e/10.1007 /s40745- 024- 00534-3 |
| 9 | Hassan et al. | 2024 | Explainable crop recommendat ion model for farmer advisory | Farmers struggle to trust black- box AI models | SHAP + ML classifier for interpret able crop advice | Improves adoption through transparenc y | Complex explanation interpretation for non-experts | https://iee explore.iee e.org/docu ment/105 66392 |
| 10 | Rahman et al. | 2023 | Smart crop recommendat ion system using IoT and ML | Lack of real- time advisory for farmers | IoT sensor based soil monitori ng with ML predictio n | Real-time decision support | Hardware and maintenance cost high | https://iee explore.iee e.org/docu ment/103 11255 |
| 11 | Chen et al. | 2025 | Transformer- based crop recommendat ion in smart farming | Traditional ML struggles with sequential climate patterns | Transfor mer AI model using soil + weather sequence s | Handles complex temporal dependencie s | High computational training cost | https://w ww.science direct.com /science/a rticle/pii/S 18770509 25019109 |
| 12 | Singh et al. | 2024 | Precision agriculture crop recommendat ion using hybrid ML models | Single- model approaches give inconsistent results | Hybrid SVM + RF recomme ndation framewo rk | Robust baseline prediction accuracy | Limited scalability for large datasets | https://iee explore.iee e.org/docu ment/104 99821 |
| 13 | Li et al. | 2025 | Deep neural network for crop recommendat ion and yield forecasting | Need integrated yield forecasting with crop selection | DNN model fused with weather and soil paramete rs | Improves yield-aware decision making | Requires long- term historical data | https://lin k.springer. com/articl e/10.1007 /s11042- 024- 20145-3 |
| 14 | Patel et al. | 2025 | Smart crop recommendat ion with remote sensing and AI | Difficulty in large-scale crop monitoring | Satellite imagery + CNN- based recomme ndation | Works for wide-area farming decisions | Remote sensing data cost is high | https://w ww.science direct.com /science/a rticle/pii/S 15698432 |

|  |  |  |  |  |  |  |  | 25001449 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Wang et al. | 2023 | Agricultural decision support for crop planning using AI | Complex crop planning under uncertain climate | Decision tree optimizat ion decision support system | Practical decision support for farmers | Limited crop validation scope | https://lin k.springer. com/articl e/10.1007 /s11356- 023- 29511-7 |
| 16 | Ahmad et al. | 2024 | AI-based crop recommendat ion using ensemble learning | Need higher accuracy in crop classificatio n | RF + XGBoost ensemble recomme ndation model | Improves classificatio n performanc e | Depends heavily on data quality | https://w ww.science direct.com /science/a rticle/pii/S 24058440 24021137 |
| 17 | Davrazos et al. | 2023 | IoT-enabled crop recommendat ion in smart agriculture using ML | Limited integration of IoT with crop advisory | IoT soil monitori ng + RF multi- class classifica tion | Real-time smart farming recommend ations | Focus mostly on accuracy, less on yield impact | https://iee explore.iee e.org/docu ment/103 45924 |
| 18 | Iniyan et al. | 2023 | Crop and fertilizer recommendat ion applying ML classifiers | Farmers need both crop and fertilizer guidance | Linear regressio n + RF + KNN recomme ndation approach | Holistic advisory system | Fertilizer module partly static | https://iee explore.iee e.org/docu ment/104 33972 |
| 19 | Zhang et al. | 2024 | AI-based smart agriculture decision support for crop selection | Need real- time crop selection support | Decision trees + IoT- based DSS platform | Improves on-farm advisory services | Requires reliable connectivity | https://iee explore.iee e.org/docu ment/104 11245 |
| 20 | Chauhan et al. | 2023 | Crop recommendat ion system based on nutrients using ML and IoT | Crop selection requires continuous soil monitoring | IoT sensors + ANN and RF classifier s | Combines live field nutrients with AI | Sensor calibration challenges | https://iee explore.iee e.org/docu ment/102 26131 |
| 21 | Senapaty et al. | 2023 | IoT-enabled soil nutrient analysis and crop recommendat ion model | Need high precision nutrient- based crop advisory | Hybrid MSVM- DAG-FFO algorith m with cloud storage | High classificatio n precision for soil- based crops | Requires continuous internet synchronizatio n | https://lin k.springer. com/articl e/10.1007 /s42979- 023- 01845-7 |
| 22 | Hossain et al. | 2023 | IoT-based smart soil fertilizer monitoring and ML crop recommendat ion | Fertilizer wastage due to lack of monitoring | Wireless sensor network + cloud ML recomme ndation | Reduces fertilizer waste and improves crop selection | Sensor installation requires expertise | https://iee explore.iee e.org/docu ment/101 00744 |

| 23 | Manoj Kumar | 2023 | ML-based crop management towards intelligent recommendat ions | Need integrated crop + pest advisory | ML predictio n + DL pest/dise ase detection module | Multifunctio nal recommend ation platform | High-quality image requirement for pest module | https://iee explore.iee e.org/docu ment/104 20050 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | Patel et al. | 2023 | Blockchain- based crop recommendat ion system for precision farming | Data trust and immutabilit y issues in farm advisory | IoT soil data + blockchai n ledger + ML recomme ndation | Ensures transparent and secure recommend ations | Blockchain adds architectural complexity | https://w ww.science direct.com /science/a rticle/pii/S 0167739X 23003892 |
| 25 | Katiyar et al. | 2025 | Cloud-based transformativ e crop recommendat ion model | Remote farmers lack timely advisory | Cloud TCRM model with SMS delivery | Accessible recommend ations even without smartphone s | Depends on stable cloud connectivity | https://lin k.springer. com/articl e/10.1007 /s42979- 025- 02418-2 |
| 26 | Behera et al. | 2025 | Federated learning for privacy- preserving crop recommendat ion | Farm data privacy limits collaborativ e model training | Federate d Learning + distribut ed DNN crop advisory | Secure learning without sharing raw data | Deployment complexity across devices | https://w ww.science direct.com /science/a rticle/pii/S 18770509 25004223 |
| 27 | Punitha et al. | 2024 | Deep learning crop recommendat ion with yield prediction model | Need accurate yield-aware crop selection | LSTM- based yield predictio n + crop recomme ndation | High predictive strength for yield planning | Computationall y heavy training process | https://w ww.science direct.com /science/a rticle/pii/S 26663074 2400038X |
| 28 | Bakr et al. | 2025 | Evaluation of ML and DL models for crop recommendat ion | Unclear best- performing models for crop advisory | Benchma rking ML vs DL models using agricultu ral datasets | Strong comparative study for researchers | Dataset- specific performance limitations | https://w ww.science direct.com /science/a rticle/pii/S 26668270 2500041X |
| 29 | Singhal et al. | 2025 | Emerging trends in crop recommendat ion systems | Fragmented research in AI crop advisory | Systemat ic review of modern CRS methods | Comprehens ive trends and research gaps | No novel implementatio n proposed | https://w ww.science direct.com /science/a rticle/pii/S 18770509 25010907 |
| 30 | Kiruthika & Karthika | 2025 | Deep learning- based crop recommendat ion using soil | Difficulty capturing sequential soil-weather patterns | LSTM sequentia l modeling for crop | Excellent temporal pattern learning | Needs large sequential datasets | https://lin k.springer. com/chapt er/10.100 7/978- |

| 31 | Alam et al. | 2025 | Crop recommendat ion with uncertainty quantification | Uncertainty in automated crop advisory decisions | Entropy- based uncertain ty model + ensemble ML | Confidence- aware crop recommend ations | Sensitive to noisy datasets | https://w ww.science direct.com /science/a rticle/pii/S 25901230 25015750 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | Baishya et al. | 2025 | TinyML based crop recommendat ion system for precision farming | Crop recommend ation on low-power edge devices | TinyML optimize d feature selection for edge deploym ent | Works offline with minimal power | Limited crop diversity in dataset | https://w ww.science direct.com /science/a rticle/pii/S 27723755 25004782 |
| 33 | Shingade et al. | 2025 | Dynamic machine learning model for crop suitability recommendat ion | Changing climatic conditions affect crop suitability | Dynamic model updating with rainfall + soil changes | Adaptable recommend ations over time | Depends on accurate historical weather data | https://lin k.springer. com/articl e/10.1007 /s11042- 025- 21014-9 |
| 34 | Reddy et al. | 2024 | Crop recommendat ion and yield prediction using hybrid ML/DL models | Need combined yield prediction with crop selection | Hybrid KNN/RF with LSTM/A NN models | Strong yield forecasting accuracy | Complex tuning and deployment | https://iee explore.iee e.org/docu ment/109 41248 |
| 35 | Zhao et al. | 2024 | Remote sensing assisted crop recommendat ion using AI models | Large-scale crop planning requires satellite intelligence | Satellite imagery + CNN recomme ndation | Supports precision agriculture at scale | High remote sensing data cost | https://w ww.science direct.com /science/a rticle/pii/S 15698432 24002336 |